Synopsis

On

AI-Enabled Smart Campus Wireless Sensor Network with Intelligent Edge Analytics

Group ID: BCAIAA22

Team Details

| Enrolment No. | Name of Student | Class |
| --- | --- | --- |
| ADT23SOCB0201 | Aryan Dhondage |  |
| ADT23SOCB0671 | Om Teke | LYCSF 3 |
| ADT23SOCB0 | Aditya Nalge |  |
| ADT23SOCB0 | Aditya Bade |  |

Under the Guidance of

Dr. Tushar Mote Sir

Department of Computer Science & Engineering

MIT School of Computing

1. Title of the Project

AI-Enabled Smart Campus Wireless Sensor Network with Intelligent Edge Analytics

2. Abstract

This project proposes an AI-enabled Smart Campus Wireless Sensor Network (WSN) for real-time environmental monitoring. Multiple ESP32-based sensor nodes equipped with DHT22 sensors continuously monitor temperature and humidity in different campus locations. Sensor data is transmitted using MQTT to a FastAPI backend, stored in a centralized database, analyzed using machine learning algorithms, and visualized on a Streamlit dashboard. The proposed solution improves monitoring ef iciency, enables intelligent anomaly detection, and provides a scalable architecture for future smart-campus deployments.

3. Introduction / Background

Educational institutions contain numerous classrooms, laboratories, libraries, seminar halls, and server rooms where environmental conditions must be monitored continuously. Traditional monitoring approaches rely on manual inspection or transmit sensor readings at ixed intervals regardless of whether any meaningful change has occurred. This increases communication overhead, storage requirements, and power consumption. The proposed system integrates IoT, AI, and cloud-ready software architecture to provide an ef icient and intelligent monitoring solution.

4. Problem Definition

Design and implement a scalable wireless sensor network capable of monitoring environmental parameters, transmitting data wirelessly, storing readings centrally, applying machine learning for anomaly detection and prediction, and presenting information through a real-time dashboard.

5. Objectives

• Design wireless sensor nodes using ESP32 and DHT22 sensors.

• Monitor temperature and humidity in multiple campus locations.

• Implement MQTT-based communication between sensor nodes and server.

• Develop a FastAPI backend and centralized database.

• Build a real-time Streamlit dashboard.

• Implement AI-based anomaly detection and prediction.

• Create a scalable and extensible smart-campus architecture.

6. Scope of the Project

The project focuses on developing an AI-enabled smart campus monitoring system that collects environmental data using ESP32 and DHT22 sensors, transmits it wirelessly via MQTT, analyzes it using AI/ML for anomaly detection, and displays real-time insights on a dashboard. The system is scalable and can be extended to smart buildings, hospitals, industries, and other IoT-based environments.

7. Proposed System

Sensor Nodes (ESP32 + DHT22)

↓

Wi-Fi Network

↓

MQTT Broker (Mosquitto)

↓

FastAPI Backend

↓

SQLite / PostgreSQL Database

↓

AI/ML Engine

↓

Streamlit Dashboard

↓

Alerts & Analytics

8. System Requirements

• Arduino IDE

• Python • FastAPI

• Mosquitto MQTT Broker

• SQLite/PostgreSQL

• Streamlit

• Scikit-learn

• Pandas

• NumPy

• Matplotlib

• Git & GitHub

Hardware Requirements :

| Component | Qty | Purpose | Approx. |
| --- | --- | --- | --- |
| ESP32 Dev Board | 4 | Sensor Node Controller | ₹2,000 |
| DHT22 Sensor | 4 | Temperature & Humidity | ₹1,000 |
| Breadboard | 4 | Prototype Circuit | ₹500 |
| Jumper Wires | 1 Pack | Connections | ₹250 |
| USB Cables | 4 | Programming & Power | ₹500 |
| USB Hub/Power | 1 | Power Supply | ₹800 |
| Miscellaneous | - | Connectors etc. | ₹500 |

9. AI Module

The AI module uses historical environmental data to detect abnormal sensor behavior and predict future trends.

• Isolation Forest for anomaly detection

• Linear Regression or Random Forest for temperature prediction

• Adaptive communication strategy (future enhancement)

• Battery life prediction (future enhancement)

10. Estimated Budget The estimated implementation cost is between ₹8,500 and ₹10,000, making the project affordable for a student team while demonstrating industry-relevant technologies.

11. Applications

• Smart Campuses

• Smart Buildings

• Hospitals

• Industrial Monitoring

• Environmental Monitoring

• Data Centres

12. Future Scope

Future work includes air-quality monitoring, occupancy detection, energy monitoring, LoRa-based communication, cloud deployment, mobile application support, and TinyML/ Edge AI on ESP32 devices.

13. Expected Outcomes

The project demonstrates an end-to-end AI-enabled IoT system capable of real-time monitoring, intelligent analytics, centralized management, and scalable deployment across smart-campus environments.

14. Conclusion

The proposed AI-Enabled Smart Campus Wireless Sensor Network integrates embedded systems, wireless communication, backend development, databases, dashboards, and machine learning into a single practical solution. The project is technically feasible, cost effective, and well aligned with the learning outcomes of a final-year B.Tech (AI & ML) program